Proj. #	MBIN-380-2(517)70--0M-07

Location	I-380 NB/SB over Elk Run Creek & Road

Desc.	Deck joint repair: Replace existing strip seal joints with new strip seal joints

(See 65-0344-506 and 43-0294-510 for sample plan references)

Nearest on-ramps: 2200 ft west of bridge, 4600 ft east of bridge

Therefore will not need traffic control at ramps

Standard Road Plans:	Notes:

BA-401	Temporary Barrier Rail (Precast Concrete)	Equiv. of IDOT’s TCB

BA-500	Temporary Crash Cushions Sand Barrel	Equiv. of IDOT’s impact attenuator

PM-110	Line Types

PM-111	Symbols and Legends	Use in advance of lane closure

PV-12	Milled Shoulder Rumble Strips	Check if rumble strips to be removed and replaced. With existing shoulder & rumble strips to remain, this should not be needed.

PV-101	Joints

SI-881	Special Signs for Workzones

TC-1	Work Not Affecting Traffic (Two-Lane or Multi-Lane)

TC-81	Restricted Width Signing (Less than 15.5 Feet)	Check and use only if width between two fixed objects is less than 15.5 Feet. With 1’-10.5” width of TBR, this should not be needed.

TC-402	Work Within 15 ft of Traveled Way

TC-418	Lane Closure on Divided Highway

TC-421	Lane Closure with TBR

TC-433	Pavement Marking Operations

(Currently matches with 65-0344-506 list; check additional sample plans)

Project Description:

This project is for bridge repairs consisting of strip seal joint replacement for the two structures over Elk Run Creek and River Road on I-380.

Index of Tabulations: Equiv. of IDOT’s sheet index but for C-sheet tables.

Figure out system for ordering of tabulation tables

Estimated Roadway Quantities: 100-0A

Project Description: 100-1D

Temporary Barrier Rail: 108-33

Temporary Crash Cushions: 108-30

Line Types: 108-22

Symbols and Legends: 108-29

Wait for Design Tabulation account access to complete list

Roadway quantities:

2527-9263109	Painted Pavement Marking, Waterborne or Solvent Based

2527-9263117	Painted Pavement Marking, Durable?

2527-9263131	Wet Removable Retroreflective Removable Tape Markings

2527-9263137	Painted Symbols and Legends, Waterborne or Solvent Based

2527-9263180	Pavement Markings, Removed

2527-9263190	Symbols and Legends, Removed

2528-8400048	Temporary Barrier Rail, Concrete

2528-8445110	Traffic Control

2528-9290050	Portable Dynamic Message Sign (PDMS)

2551-0000110	Temporary Crash Cushion

For pmk quantities measured in STA, divide LF quantity by 100. For 6-inch lines, multiply by factor of 1.5